DESIGN AND SIMULATION OF A FOUR-WAY TRAFFIC LIGHT

CONTROL SYSTEM USING PLC LADDER LOGIC

Siemens SIMATIC S7-300 (CPU 313C) | STEP 7 Ladder Logic (LAD)

Submitted by

Dibyashree Bijayeeni Sahu

Registration number: 2302050053

Department: Electrical Engineering

Veer Surendra Sai University of Technology, Burla

July,2026

# Table of Contents

1. Abstract

2. Objective

3. Introduction

4. Platform and Tools Used

5. System Description and I/O List

6. Design Methodology

7. Phase Sequence and Timing Chart

8. Results and Discussion

9. Conclusion

10. Future Scope

11. References

# 1. Abstract

This report documents the design of a four-way traffic light control system implemented in PLC ladder logic on a Siemens SIMATIC S7-300 (CPU 313C) platform, programmed in STEP 7. The controller manages red, yellow, and green signal lamps for four approach roads (East, North, West, and South) at a single intersection, using a chain of eight 5-second on-delay timers as a sequencer. The system is started and stopped through a latched pushbutton circuit and cycles continuously through eight timed phases, giving each road one green interval and two yellow (transition) intervals per 40-second cycle. This project demonstrates core PLC concepts: digital I/O mapping, latching/seal-in circuits, on-delay timer instructions, and timer-chained sequencing - all without requiring a dedicated sequencer or drum instruction.

# 2. Objective

- To design a ladder logic program that automatically controls red, yellow, and green signals for a four-way road intersection.

- To implement a Start/Stop latch circuit so the sequence can be safely enabled and halted by an operator.

- To build a repeatable timing sequence using chained on-delay timers instead of a dedicated sequencer instruction.

- To document the input/output mapping, network-by-network logic, and phase timing for reuse, review, and portfolio presentation.

# 3. Introduction

## 3.1 What is a PLC?

A Programmable Logic Controller (PLC) is an industrial computer that continuously reads field inputs (pushbuttons, sensors), executes a control program, and drives field outputs (lamps, motors, valves, relays). It operates in a repeating scan cycle: read inputs, execute the program from top to bottom, update outputs, and repeat - typically many times per second - for as long as the controller is powered.

## 3.2 What is Ladder Logic?

Ladder logic is a graphical PLC programming language modeled on electrical relay schematics. A program is organized into rungs (called Networks in Siemens STEP 7), each containing contacts (conditions, similar to switches) that control a coil (an output) at the end of the rung. Contacts wired in series behave as a logical AND; contacts wired in parallel behave as a logical OR.

## 3.3 Problem Statement

A four-way intersection needs a controller that lets exactly one approach road proceed on green at a time, gives a clear yellow warning before that road stops, and keeps every other approach on red - repeating indefinitely until an operator stops the system.

# 4. Platform and Tools Used

- Controller: Siemens SIMATIC S7-300, CPU 313C

- Programming software: Siemens STEP 7 (SIMATIC Manager), Ladder (LAD) editor

- Organization block: OB1 - Main Program Sweep (Cycle)

- Programming language: Ladder Logic (LAD), IEC 61131-3 style

# 5. System Description and I/O List

The system uses two physical inputs, one internal memory bit, twelve physical outputs (three lamps per road), and eight internal timers. The full address list is given below.

| Address | Signal | Type |
| --- | --- | --- |
| I0.0 | Start pushbutton | Input |
| I0.1 | Stop pushbutton | Input |
| M0.0 | System-running flag (latch) | Internal memory |
| Q0.0 | East - Green lamp | Output |
| Q0.1 | North - Red lamp | Output |
| Q0.2 | West - Red lamp | Output |
| Q0.3 | South - Red lamp | Output |
| Q0.4 | East - Yellow lamp | Output |
| Q0.5 | North - Yellow lamp | Output |
| Q0.6 | North - Green lamp | Output |
| Q0.7 | East - Red lamp | Output |
| Q1.0 | West - Yellow lamp | Output |
| Q1.1 | West - Green lamp | Output |
| Q1.2 | South - Yellow lamp | Output |
| Q1.3 | South - Green lamp | Output |
| T0 - T7 | Eight 5-second on-delay timers forming the sequencer | Timer |

# 6. Design Methodology

## 6.1 Start/Stop Latch (Network 1)

The first network implements a standard seal-in (latching) circuit. Pressing the Start input (I0.0) sets the memory bit M0.0. Because M0.0 feeds back into its own enabling condition, it remains on after the Start button is released, and stays on until the Stop input (I0.1) is pressed. Every other network in the program is conditioned on M0.0, so no timer runs and no lamp changes state unless the system has been started.

## 6.2 Timer-Chained Sequencing (Networks 2-8)

Rather than using a dedicated sequencer instruction, the design chains eight identical 5-second on-delay timers (T0-T7) so that each timer's done bit is the starting condition for the next timer: T0 done starts T1, T1 done starts T2, and so on through T7, whose done bit re-starts T0 and restarts the whole cycle. Each timer therefore owns one 5-second phase of a 40-second repeating cycle, and the lamps that must be lit during that phase are wired to activate for exactly as long as that timer is timing.

Each timer block used is a Siemens on-delay timer (S_ODT for T0, and the retentive S_ODTS variant for T1-T7), with six standard connections:

- S (Start) - the condition that begins the timing.

- TV (Time preset) - set to S5T#5S, five seconds, on every timer in this program.

- R (Reset) - forces the timer back to zero.

- Q (Output) - switches to 1 once the elapsed time reaches the preset; this done bit is what triggers the next network.

- BI / BCD - the current elapsed time as a raw number or display-ready digits (used for monitoring, not for control, in this program).

## 6.3 Network Summary

The table below summarizes what condition fires each network, which timer it starts, which lamps it drives, and which phase of the cycle (see Section 7) that corresponds to.

| Network | Fires when | Starts | Outputs driven | Phase |
| --- | --- | --- | --- | --- |
| 1 | I0.0 pressed, or M0.0 already on, AND I0.1 not pressed | - | M0.0 (latch) | - |
| 2 | M0.0 on AND T7 done | T0 | East Green; North/West/South Red | 1 |
| 3 | M0.0 on AND T0 done | T1 | East Yellow; North Yellow | 2 |
| 4 | M0.0 on AND T1 done | T2 | North Green; East Red | 3 |
| 5 | M0.0 on AND T2 done | T3 | West Yellow; North Yellow (2nd) | 4 |
| 6 | M0.0 on AND T3 done | T4 | West Green | 5 |
| 7 | M0.0 on AND T4 done; also T5 done | T5 | South Yellow; West Yellow (2nd); South Green | 6, 7 |
| 8 | M0.0 on AND T5 done; then T6 done | T6, T7 | South Yellow (2nd); East Yellow (2nd) | 7, 8 |

# 7. Phase Sequence and Timing Chart

Because the design hands off a single rotating green from East to North to West to South and back to East, each road receives one 5-second green interval, two 5-second yellow intervals (one handing its green off to the next road, one receiving the hand-off from the previous road), and 25 seconds of red - for a 40-second total cycle. The complete sequence is:

| Phase | Timer | East | North | West | South |
| --- | --- | --- | --- | --- | --- |
| 1 | T0 | GREEN | Red | Red | Red |
| 2 | T1 | Yellow | Yellow | Red | Red |
| 3 | T2 | Red | GREEN | Red | Red |
| 4 | T3 | Red | Yellow | Yellow | Red |
| 5 | T4 | Red | Red | GREEN | Red |
| 6 | T5 | Red | Red | Yellow | Yellow |
| 7 | T6 | Red | Red | Red | GREEN |
| 8 | T7 | Yellow | Red | Red | Yellow |

Design note: real-world intersections more commonly pair opposite approaches together (North-South on one phase, East-West on the other) and give each road a single trailing yellow rather than a leading one. The rotating single-green pattern used here was chosen because it cleanly demonstrates the timer-daisy-chain technique with the fewest timers; it can be adapted to a paired-phase layout by combining two roads' outputs on each rung and reducing the cycle to four phases.

# 8. Results and Discussion

When simulated, pressing Start energizes M0.0 and immediately begins Phase 1 (East green). The controller then advances automatically through all eight phases at 5-second intervals, with exactly one road green at any instant and every transition covered by a shared yellow warning period. Pressing Stop at any point freezes the outputs in their current state; pressing Start again resumes the latch, and the cycle continues from wherever the timer chain left off. The design confirms that a full multi-phase sequence can be built reliably from ordinary on-delay timers without a dedicated sequencer/drum instruction, at the cost of needing OR-branches on any output (such as the red lamps) that must stay on across more than one phase.

# 9. Conclusion

This project implements a complete, self-restarting four-way traffic light controller in Siemens STEP 7 ladder logic, using a latched Start/Stop circuit and a chain of eight on-delay timers as the sequencing mechanism. It demonstrates practical PLC fundamentals - digital I/O mapping, seal-in circuits, and timer-based sequencing - in a compact, easily simulated program that is well suited as an introductory PLC programming exercise.

# 10. Future Scope

- Add pedestrian crossing signals and push-to-cross inputs for each road.

- Replace fixed 5-second timing with adjustable presets or traffic-density-based (sensor-driven) timing.

- Extend the design to paired opposite-road phases (North-South / East-West) for a more realistic intersection.

- Add a SCADA/HMI screen to visualize live signal states and allow remote start/stop.

- Add a manual/flashing-amber fallback mode for maintenance or power-up conditions.

# 11. References

1. Siemens SIMATIC STEP 7 documentation - Programmable Logic Controllers, on-delay timer (S_ODT/S_ODTS) instructions.

2. Automation Community, "Traffic Light Control using PLC Ladder Logic Programming" (reference tutorial on four-way traffic light sequencing logic).

3. Project ladder logic export: SIMATIC test004 \ SIMATIC 300(1) \ CPU 313C \ OB1, generated from the STEP 7 project used for this report.